# Smart Campus Energy Management System

Technical Specifications and Design Documents

# Document Version: 1.0

Last Updated: November 20, 2025

Project Name: Smart Campus Energy Management System

Document Owner: Priyanka Sharma

Project Team: Kayode Babalola, Qianru Deng, Priyanka Sharma

Organization: St. Clair College

Executive Summary

This document provides comprehensive technical specifications and design requirements for the Smart Campus Energy Management System. The system aims to reduce campus energy consumption by 15% through real-time monitoring, data analytics, and behavioral change initiatives.

Table of Contents

- System Architecture Overview

- Hardware Specifications

- Software Requirements and Technology Stack

- Dashboard Features and Requirements

- Data Integration Specifications

- Network Architecture

- Security and Compliance Requirements

- Implementation Timeline

- Quality Assurance and Testing

- System Architecture Overview

  - High-Level System Design

The Smart Campus Energy Management System consists of three interconnected layers designed for scalability, reliability, and user accessibility.

# Layer 1: Data Collection (IoT Sensor Network)

IoT energy sensors deployed across campus buildings

Real-time energy consumption monitoring at building and department levels Wireless communication protocols (WiFi 802.11n)

Local data buffering for network resilience

Data transmission every 15 minutes to central server

# Layer 2: Data Processing and Analytics Platform

Cloud-based data aggregation and storage (AWS/Azure) Real-time data processing pipeline

Historical data analysis and trend identification Machine learning models for consumption predictions RESTful API for secure data retrieval

Automated anomaly detection system

# Layer 3: User Interface and Engagement

Web-based administrative dashboard Mobile application for students and staff Educational campaign management portal Real-time energy consumption visualization Gamification and engagement features

  - System Integration Points

Campus Building Management System (BMS): Integration for HVAC and lighting control

Student Information System: User authentication and profile management

Weather API: External weather data for contextual analysis

Campus Directory: Department and building information

- Hardware Specifications

  - Energy Monitoring Sensors

| Specification | amp; Details |
| --- | --- |
| Model | amp; Smart Energy Meter (SEM-3000) |
| Measurement Range | amp; 0-999.99 kWh |
| Accuracy | amp; ±0.5% at nominal load |
| Communication Protocol | amp; WiFi 802.11n, 2.4 GHz |
| Power Supply | amp; AC 230V with 12-hour battery backup |
| Operating Temperature | amp; -10°C to +60°C |
| Installation Location | amp; Main electrical panels, building entrances |
| Quantity Required | amp; 28 sensors (St. Clair College campus) |
| Expected Lifespan | amp; 10 years (manufacturer warranty: 5 years) |
| Certifications | amp; UL Listed, CSA Approved, Energy Star |

  - Data Transmission Gateway

| Specification | amp; Details |
| --- | --- |
| Communication Protocol Transmission Frequency Data Format Local Storage Capacity Connectivity Options Processing Capacity Redundancy | amp; MQTT over TLS, HTTP/HTTPS amp; Every 15 minutes (configurable) amp; JSON structured format amp; 30 days of buffered data amp; Ethernet (primary), 4G LTE (backup) amp; 100 sensors per gateway amp; Dual gateway deployment |

  - Server Infrastructure Requirements

Cloud Platform: AWS (Amazon Web Services) or Microsoft Azure

# Primary Server Specifications:

Instance Type: t3.large (or equivalent) CPU: 4 vCPUs, 2.5 GHz Intel Xeon RAM: 16 GB DDR4

Storage: 1 TB SSD (scalable to 5 TB) Network: 10 Gbps bandwidth

Uptime SLA: 99.9%

# Database Server:

Database: PostgreSQL 14.x Replication: Master-slave configuration

Backup Schedule: Automated daily backups with 30-day retention Disaster Recovery: Cross-region backup replication

- Software Requirements and Technology Stack

  - Backend Technology Stack

Programming Language: Python 3.10+

Web Framework: Django 4.2 or FastAPI 0.100+

# Database:

Primary: PostgreSQL 14.x Caching: Redis 7.x

Time-series data: InfluxDB 2.x

Message Queue: RabbitMQ 3.12 or Apache Kafka

API Design: RESTful API with OpenAPI 3.0 documentation

Authentication: OAuth 2.0, JWT tokens

  - Frontend Technology Stack

# Web Application:

Framework: React.js 18+ with TypeScript UI Library: Material-UI (MUI) v5

State Management: Redux Toolkit Charting: Chart.js 4.x, D3.js v7 Build Tool: Vite 4.x

# Mobile Application:

Framework: React Native 0.72+

Target Platforms: iOS 14+ and Android 10+ Navigation: React Navigation v6

Offline Support: AsyncStorage, Redux Persist

  - DevOps and Infrastructure

Containerization: Docker 24.x Orchestration: Kubernetes 1.28 CI/CD Pipeline: GitHub Actions Monitoring and Logging:

Application Monitoring: Prometheus Visualization: Grafana

Log Aggregation: ELK Stack (Elasticsearch, Logstash, Kibana) Error Tracking: Sentry

Infrastructure as Code: Terraform

- Dashboard Features and Requirements

  - Administrator Dashboard

# Core Features:

    - Real-Time Energy Monitoring

Campus-wide energy consumption display

Building-level breakdown with interactive map  Department-level granular views

Historical comparison (day, week, month, year)

# Data Visualization

Line charts for consumption trends  Bar charts for comparative analysis

Heat maps for building occupancy vs. energy use  Pie charts for energy source distribution

# Anomaly Detection and Alerts

Automated detection of unusual consumption patterns  Email and SMS notifications for anomalies

Threshold-based alert configuration

Alert acknowledgment and tracking system

# Report Generation

Automated weekly and monthly reports

Custom report builder with date range selection  Export formats: PDF, Excel, CSV

Scheduled report delivery via email

# User Management

Role-based access control (Admin, Manager, Viewer)  User activity logging

Permission management  Authentication audit trail

# Energy Efficiency Recommendations

AI-powered optimization suggestions  ROI calculator for efficiency measures  Best practice library

Implementation tracking

# Performance Requirements:

Page load time: < 2 seconds on standard broadband Dashboard refresh rate: Every 30 seconds (configurable) Concurrent user support: 500+ simultaneous users

Data latency: < 1 minute from sensor to dashboard Mobile responsiveness: Fully responsive across devices

  - Student and Staff Mobile Application

# Core Features:

    - Personal Dashboard

Campus-wide energy status overview  Personal energy impact tracking

Building-specific consumption data  Weekly consumption summaries

# Educational Content

Energy-saving tips and best practices  Video tutorials on sustainability

Interactive quizzes and challenges

News feed for campus energy initiatives

# Gamification Elements

Personal energy savings score  Department leaderboards

Achievement badges and rewards  Social sharing capabilities

# Notifications

Push notifications for energy campaigns  Alerts for high-consumption periods

Reminder notifications for energy challenges  Event announcements

# Technical Requirements:

Platform Compatibility: iOS 14+ and Android 10+ App Size: < 50 MB download size

Offline Functionality: Last 7 days of personal data Battery Impact: < 2% battery usage per day Accessibility: WCAG 2.1 AA compliant

- Data Integration Specifications

  - Data Collection Points

# Primary Data Sources:

Building-level energy metering (28 locations) Department sub-metering (15 major departments) HVAC system monitoring

Lighting control systems

Weather data (external API integration)

  - API Endpoints

| Endpoint | amp; Method | amp; Purpose |
| --- | --- | --- |
| /api/v1/sensors | amp; GET | amp; List all active sensors |
| /api/v1/sensors/{id} | amp; GET | amp; Get specific sensor details |
| /api/v1/data/current | amp; GET | amp; Real-time consumption data |
| /api/v1/data/history | amp; GET | amp; Historical data with filters |
| /api/v1/predictions | amp; GET | amp; Energy forecasts |
| /api/v1/alerts | amp; GET, POST | amp; Alert management |
| /api/v1/reports | amp; GET, POST | amp; Report generation |
| /api/v1/users | amp; GET, POST | amp; User management |

  - Data Retention Policy

Real-time data: 90 days at full resolution (15-minute intervals)

Historical data: 5 years at hourly aggregation Archival data: Indefinite retention at daily aggregation User activity logs: 2 years retention

Backup retention: 30-day rolling backup

- Network Architecture

  - Network Topology

# Sensor Network:

Sensors → WiFi Access Points → Campus Network → Gateway Server → Cloud Platform

# Redundancy and Fault Tolerance:

Dual gateway deployment for redundancy Multiple WiFi access points per building Automatic failover to 4G LTE backup

Local data buffering during network outages

  - Bandwidth Requirements

# Per Sensor:

Data payload size: ~5 KB per transmission

Transmission frequency: Every 15 minutes (96 times/day) Daily bandwidth per sensor: ~480 KB

# Total Campus Network:

28 sensors × 480 KB = 13.44 MB/day Average bandwidth: 2-5 Mbps

Peak bandwidth: 10 Mbps (during system synchronization)

  - Network Security

Encryption in Transit: TLS 1.3 or higher for all data transmission VPN Tunnels: Secure connections between sensors and cloud Firewall Configuration: Whitelist-based access control

Network Segmentation: Isolated VLAN for IoT devices

Intrusion Detection: 24/7 monitoring for suspicious activity

- Security and Compliance Requirements

  - Data Security

# Encryption Standards:

Data in Transit: TLS 1.3

Data at Rest: AES-256 encryption

Database Encryption: Transparent Data Encryption (TDE) Backup Encryption: AES-256

# Authentication and Authorization:

Multi-factor authentication (MFA) for admin accounts OAuth 2.0 and JWT for API authentication

Role-based access control (RBAC) Session timeout: 30 minutes of inactivity

Password policy: Minimum 12 characters, complexity requirements

# API Security:

Rate limiting: 100 requests per minute per user API key rotation: Every 90 days

Request validation and sanitization CORS policy enforcement

  - Privacy and Compliance

# Data Privacy:

Compliance with GDPR and PIPEDA (Canadian privacy law) Anonymization of personal user data

Data minimization principles User consent management

Right to data deletion

# Accessibility:

WCAG 2.1 Level AA compliance Screen reader compatibility Keyboard navigation support Color contrast requirements Alternative text for images

# Audit and Logging:

All system actions logged with timestamps User activity audit trail

Admin action logging Log retention: 2 years

Security event monitoring and alerting

- Quality Assurance and Testing

  - Testing Strategy

# Unit Testing:

Code coverage target: 80%

Automated test execution in CI/CD pipeline

Test frameworks: pytest (Python), Jest (JavaScript)

# Integration Testing:

API endpoint testing Database integration testing

Third-party service integration testing

# User Acceptance Testing (UAT):

Test group: 50 students, staff, and administrators Duration: 5 days (Nov 25-30)

Success criteria: 90% user satisfaction

# Performance Testing:

Load testing: 1000 concurrent users Stress testing: 2x expected load

Latency testing: < 1 second response time

# Security Testing:

Penetration testing by third-party security firm Vulnerability scanning (OWASP Top 10) Code security analysis

Appendix A: Glossary of Terms

API: Application Programming Interface

IoT: Internet of Things

kWh: Kilowatt-hour (unit of energy)

MQTT: Message Queuing Telemetry Transport

REST: Representational State Transfer

TLS: Transport Layer Security

Document Classification: Internal Use Only Next Review Date: December 15, 2025 Approved By: Project Team

Contact: priyanka.sharma@stclaircollege.ca